Descriptions des fonctions C de la bibliothèque WiringPi écrites par M. Gordon Hendersons.

## Functions (API)

Some of the functions in the WiringPi library are designed to mimic those in the Arduino Wiring system. There are relatively easy to use and should present no problems for anyone used to the Arduino system, or C programming in-general.

The main difference is that unlike the Arduino system, the main loop of the program is not provided for you – you need to write it yourself. This is often desirable in a Linux system anyway as it can give you access to command-line arguments and so on. Before using the WiringPi library, you need to include its header file:

#include <wiringPi.h>

You may also need to add

-I/usr/local/include -L/usr/local/lib -lwiringPi

to the compile line of your program depending on the environment you are using. The important one is -lwiringPi

## Setup Functions

There are three ways to initialise wiringPi.

- int wiringPiSetup (void) ;

- int wiringPiSetupGpio (void) ;

- int wiringPiSetupSys (void) ;

One of the setup functions must be called at the start of your program. If it returns -1 then the initialisation of the GPIO has failed, and you should consult the global errno to see why.

The differences between the setup functions are as follows:

- wiringPiSetup(void) ;

This initialises the wiringPi system and assumes that the calling program is going to be using the wiringPi pin numbering scheme. This is a simplified numbering scheme which provides a mapping from virtual pin numbers 0 through 16 to the real underlying Broadcom GPIO pin numbers. See the pins page for a table which maps the wiringPi pin number to the Broadcom GPIO pin number to the physical location on the edge connector.

This function needs to be called with root privileges :

- wiringPiSetupGpio(void) ;

This is identical to above, however it allows the calling programs to use the Broadcom GPIO pin numbers directly with no re-mapping.

As above, this function need to be called with root priveledges :

- wiringPiSetupSys(void)

This initialises the wiringPi system but uses the /sys/class/gpio interface rather than accessing the hardware directly. This can be called as a non-root user provided the GPIO pins have been exported before-hand using the gpio program. Pin number in this mode is the native Broadcom GPIO numbers.

Note: In this mode you can only use the pins which have been exported via the /sys/class/gpio interface. You must export these pins before you call your program. You can do this in a separate shell-script, or by using the system() function from inside your program.

Also note that some functions (noted below) have no effect when using this mode as they’re not currently possible to action unless called with root privileges .

## General wiring functions

- void pinMode (int pin, int mode) ;

This sets the mode of a pin to either INPUT, OUTPUT, or PWM_OUTPUT. Note that only wiringPi pin 1 (BCM_GPIO 18) supports PWM output. The pin number is the number obtained from the pins table.

This function has no effect when in Sys mode.

- void digitalWrite (int pin, int value) ;

Writes the value HIGH or LOW (1 or 0) to the given pin which must have been previously set as an output.

- void pwmWrite (int pin, int value) ;

Writes the value to the PWM register for the given pin. The value must be between 0 and 1024. (Again, note that only pin 1 (BCM_GPIO 18) supports PWM)

This function has no effect when in Sys mode (see above) :

- int digitalRead (int pin) ;

This function returns the value read at the given pin. It will be HIGH or LOW (1 or 0) depending on the logic level at the pin.

- void pullUpDnControl (int pin, int pud) ;

This sets the pull-up or pull-down resistor mode on the given pin, which should be set as an input. Unlike the Arduino, the BCM2835 has both pull-up an down internal resistors. The parameter pud should be; PUD_OFF, (no pull up/down), PUD_DOWN (pull to ground) or PUD_UP (pull to 3.3v)

This function has no effect when in Sys mode. If you need to activate a pull-up/pull-down, then you can do it with the gpio program in a script before you start your program.

## PWM Control

PWM can not be controlled when running in Sys mode.

- pwmSetMode (int mode) ;

The PWM generator can run in 2 modes – “balanced” and “mark:space”. The mark:space mode is traditional, however the default mode in the Pi is “balanced”. You can switch modes by supplying the parameter: PWM_MODE_BAL or PWM_MODE_MS.

- pwmSetRange (unsigned int range) ;

This sets the range register in the PWM generator. The default is 1024.

- pwmSetClock (int divisor) ;

This sets the divisor for the PWM clock.

To understand more about the PWM system, you’ll need to read the Broadcom ARM peripherals manual.

## Timing functions

- unsigned int millis (void)

This returns a number representing the number if milliseconds since your program called one of the wiringPiSetup functions. It returns an unsigned 32-bit number which wraps after 49 days.

- void delay (unsigned int howLong)

This causes program execution to pause for at least howLong milliseconds. Due to the multi-tasking nature of Linux it could be longer. Note that the maximum delay is an unsigned 32-bit integer or approximately 49 days.

- void delayMicroseconds (unsigned int howLong)

This causes program execution to pause for at least howLong microseconds. Due to the multi-tasking nature of Linux it could be longer. Note that the maximum delay is an unsigned 32-bit integer microseconds or approximately 71 minutes.

## LCD Library

Part of wiringPi is a library to allow access to parallel interface LCD displays (Those that use the popular Hitachi HD44780U or compatible controllers)

The library is simple to use in your own programs, however wiring the displays up may be challenging, so do take care. It is possible to wire up more than one display! In 8-bit mode, the first display needs 10 GPIO pins and each additional display needs just one more pin, so with a maximum of 17 GPIO pins, that’s 8 displays. If you move to using a 4-bit interface (trivial in the code), then it’s 4 more displays – 12 LCDs! However I suspect the rest of the wiring might be somewhat challenging… Wiring is described at the end of the this page.

The LCD display can be either a 5V display or a 3,3v display, however if we are using a 5V display then we must make absolutely sure the display can never write data back to the Raspberry Pi, otherwise it will present 5V on the Pi’s GPIO pins which will not be good. At best you’ll destroy the pin drivers, at worst you’ll destroy your Pi.

So make sure you always connect the R/W pin on the display to ground to force the display to be read-only to the host.

To use the LCD library, you’ll need this at the start of your program:

#include <wiringPi.h>

#include <lcd.h>

First, you need to initialise wiringPi in the way you want to. The LCD library will call pinMode functions, but these are ignored if you have already set the modes using the gpio program and want to use the wiringPiSetupSys() mechanism.

- int  lcdInit (int rows, int cols, int bits, int rs, int strb,
        int d0, int d1, int d2, int d3, int d4, int d5, int d6, int d7) ;

This is the main initialisation function and must be called before you use any other LCD functions.

Rows and cols are the rows and columns on the display (e.g. 2, 16 or 4,20). Bits is the number of bits wide on the interface (4 or 8). The rs and strb represent the pin numbers of the displays RS pin and Strobe (E) pin. The parameters d0 through d7 are the pin numbers of the 8 data pins connected from the Pi to the display. Only the first 4 are used if you are running the display in 4-bit mode.

The pin numbers will be either wiringPi pin numbersof GPIO pin numbers depending on which wiringPiSetup function you used.

The return value is the ‘handle’ to be used for all subsequent calls to the lcd library when dealing with that LCD, or -1 to indicate a fault (Usually incorrect parameters).

LCD in  4-bit mode would be initialised by:

fd = lcdInit (2, 16, 4,  11,10 , 0,1,2,3,0,0,0,0) ;

and LCD in  8-bit mode by:

fd = lcdInit (2, 16, 8,  11,10 , 0,1,2,3,4,5,6,7) ;

- lcdHome (int handle)

- lcdClear (int handle)

These home the cursor and clear the screen respectively.

- lcdPosition (int handle, int x, int y)

Set the position of the cursor for subsequent text entry.

- lcdPutchar (int handle, uint8_t data)

- lcdPuts (int handle, char *string)

- lcdPrintf (int handle, char *message, …)

These output a single ASCII character, a string or a formatted string using the usual printf formatting commands.

At the moment, there is no input cursor control and no clever scrolling or wrapping of data sent to the display. It is up to your program to manage the display as it needs to.

Do see an example program in the examples directory inside the wiringPi software distribution.